LI-600 Porometer/Fluorometer

Links of interest: Measuring Stomatal Conductance and Chlorophyll Fluorescence with the New LI-600 Porometer/Fluorometer (confex.com)

The porometer uses a mass balance for water vapor flux from the leaf to compute stomatal conductance. The fluorometer uses optical techniques to probe the quantum yield of photosystem II.

As plantas, ao absorverem CO2, inevitavelmente, perdem água pelas folhas. Essa perda de água ocorre principalmente através dos estômatos, que apresentam mecanismos para controlar o seu grau de abertura. Esse controle é atribuído à condutância estomática foliar, que é frequentemente utilizada como indicador da deficiência hídrica.

The LI-600 also has a GPS receiver and an accel-erometer/magnetometer that measures the pitch, roll, and heading of a leaf.

Porometer (conductance)

Stomatal conductance (gsw) is computed from transpiration (E) in a leaf cuvette and leaf temperature (Tleaf) is measured by a noncontact infrared thermometer.

gsw - Stomatal conductance

gbw - One-sided boundary layer conductance

gtw - Total conductance

E_apparent - Transpiration

VPcham - Chamber vapor pressure

VPref - Reference vapor pressure

VPleaf - Leaf vapor pressure

VPDleaf - Vapor pressure deficit

H2O_r - Reference H2O mole fraction

H2O_s - Chamber H2O mole fraction

H2O_leaf - Leaf H2O mole fraction

leaf_area - Leaf area in cm2

Fluorometry (photosystem II)

flashID - Flash file ID

Fo - Minimum fluorescence in dark

Fm - Maximum fluorescence in dark

Fv/Fm - Quantum efficiency in dark

Fs - Minimum fluorescence in light

Fm' - Maximum fluorescence in light

PhiPS2 - Quantum efficiency in light

PS2/1 - Ratio of PSII to PSII absorptance

Abs - Leaf light absorptance

ETR - Electron transport rate

Sensors

The Sensor group contains the values recorded by the relative humidity, temperature, flow, pressure, and PAR sensors as well as the battery voltage.

rh_s - Sample relative humidity

rh_r - Reference relative humidity

Tref - Block reference temperature

Tleaf - IRT leaf (sample) temperature

P_atm - Atmospheric pressure

Flow - Flow rate to cuvette

flow_s - Exhaust flow leaving cuvette

leak_pct - Flow percent leak

Qamb - Ambient light

batt - Battery voltage

Leaf angle

The leaf angle group contains measurements from the magnetometer, accelerometer, GPS, and calculated variables.

pitch - Slope from horizontal

roll - Rotation from horizontal

heading - Rotation from North

angle_inc_leaf - Angle of incidence between the leaf and the sun

direct_pct - Proportion of direct irradiance incident on leaf

slope_leaf - Leaf inclination from horizontal

az_leaf - Leaf rotation from south

dec_solar - Angular position of sun with respect to

equator

az_solar - Solar rotation

zenith_solar - Solar zenith angle

Match

The Match group records the time and date of the RH sensor matching and the sensor correction.

match_time - Match time

match_date - Match date

rh_adj - Relative humidity sensor correction

Stability

The Stability group records the measurement stability of stomatal conductance(gsw), and slope and fluorescence (flr) at 1, 2, and 4 seconds.

gsw1sec - Stomatal conductance at 1 sec

gsw2sec - Stomatal conductance at 2 sec

gsw4sec - Stomatal conductance at 4 sec

flr1sec - Slope and fluorescence at 1 sec

flr2sec - Slope and fluorescence at 2 sec

flr4sec - Slope and fluorescence at 4 sec

Porometer configuration

auto - Auto mode

flow_set - Flow set point

gsw_limit - gsw slope limit

gsw_period - gsw period

Fluorometer configuration

Dark - Dark adapted or not

flash_type - Flash type

flash_intensity - Flash intensity

modrate - Modulation rate

flr_limit - Flr slope limit

flr_period - Flr period

P1_dur - MPF phase 1 duration

P2_dur - MPF phase 2 duration

P3_dur - MPF phase 3 duration

P1_Fmax - Phase 1 maximum fluorescence

P2_ramp - Phase 2 ramp depth

P2_slp - Phase 2 regression slope

P3_Fmax - Phase 3 maximum fluorescence

P3_Pred - Phase 3 predicted fluorescence

P3_DeltaF - Phase 3 measured–phase 3 predicted

Sensor voltage

The Sensor Voltage group reports the voltage for each of the sensors. The output in the SENSOR group is calculated from the SENSOR_V values and calibration coefficients determined at the factory.

v_humA - Reference RH sensor (poro)

v_humB - Sample RH sensor (poro)

v_flowIn - Inlet flow sensor (poro)

v_flowOut - Outlet flow sensor (poro)

v_temp - Thermistor (poro)

v_irt - IRT target temperature (poro)

v_pres - Pressure sensor (poro)

v_par - Quantum sensor (poro)

v_F - De-modulated fluorescence (fluoro)

i_LED - LED current (optional) (fluoro)

User calibrations

b_rhr - Ref RH sensor zero intercept (poro)

m_rhr - Ref RH sensor slope (poro)

span_rhr - Ref RH sensor span (poro)

b_rhs - Sam RH sensor zero intercept (poro)

m_rhs - Sam RH sensor zero slope (poro)

span_rhs - Sam RH sensor span (poro)

z_flowIn - Flow in zero (poro)

z_flowOut - Flow out zero (poro)

z_quantum - Quantum sensor zero (poro)

z_flr - Fluor zero (optional) (fluor)

Metadata

The metadata for each measurement includes instrument and sensor serial numbers, embedded firmware version, and when the configuration was last updated.

lashId - Flash ID (fluoro)

lciSerNum - Instrument serial#

lcpSerNum - Porometer serial#

lcfSerNum - Fluorometer serial#

lcrhSerNum - RH sensor serial number

version - Embedded version

configUpdatedAt - Configuration update date

The total amount of light incident on the leaf from the LEDs is then calculated from:

PARflr is written to flash files during each log point..